МІНІСТЕРСТВО ОСВІТИ І НАУКИ УКРАЇНИ

НАЦІОНАЛЬНИЙ УНІВЕРСИТЕТ «ЛЬВІВСЬКА ПОЛІТЕХНІКА»

Інститут комп’ютерних технологій, автоматики та метрології

Кафедра комп’ютеризованих систем автоматики

Лабораторна робота №2

З Дисципліни “програмно-апаратне забезпечення систем рухомих об’єктів”

Виконав Сясько Ю.

Студент групи ІР-33, варіант 23

Прийняв Шихмат А.О.

Львів 2026

Моделювання безпілотних систем у Gazebo та ArduPilot SITL. Створення і тестування алгоритмів керування

Дата виконання: 22.10.2026

Мета роботи: розгорнути середовище моделювання Gazebo Sim + ArduPilot SITL + ROS 2, запустити приклад польотного тесту, реалізувати власний алгоритм польоту до цільової точки за варіантом, автоматично посадити дрон і оцінити точність.

ЗАВДАННЯ:

Індивідуальне завдання (варіант 23): долетіти з початкової точки (x0, y0) до точки x_target = x0 − Δx, y_target = y0 − Δy, де Δx = 5.2 м, Δy = 0.8 м (відстань ≈ 5.26 м). Допуск 0.2–0.3 м; на підльоті швидкість зменшується, горизонтальний рух зупиняється, далі виконується автоматична посадка і коректне завершення програми. Зафіксувати початкову, цільову та фактичну координати перед посадкою, похибку e = √((x_факт − x_ціль)² + (y_факт − y_ціль)²), результат посадки, лог і два скріншоти.

Документація:

Github - “https://github.com/YuraSiasko/lab2_hardwareAndSoftwareForMobileObjectSystems”

ArduPilot Gazebo / ROS 2: https://ardupilot.org/dev/docs/ros2-gazebo.html ; методичні вказівки до лабораторної роботи №2.

Реалізація:

1. Операційна система та програмне забезпечення

| Компонент | Версія |
| --- | --- |
| Хост-ОС | Windows 10 (22H2, збірка 19045), WSL 3.0.3 (WSL2) |
| Середовище виконання | Ubuntu 22.04.5 LTS (окремий дистрибутив WSL2 «Ubuntu-22.04» на диску D:), ядро 6.18.54.1-microsoft-standard-WSL2 |
| ROS 2 | Humble Hawksbill (ros-base 0.10.0) |
| Gazebo Sim | Harmonic (gz-sim 8.15.0) |
| ArduPilot SITL | ArduCopter V4.8.0-dev (commit 3d24375e57), модель JSON |
| Інтеграція | ardupilot_gz, ardupilot_gazebo, micro-ROS agent 3.0.6 (DDS, UDP 2019), ros_gz 0.244.27, sdformat_urdf (гілка humble) |
| Інше | Python 3.10.12, colcon-core 0.21.3, MAVProxy |
| Обладнання | ПК 16 ГБ ОЗУ (WSL2 виділено ≈7.8 ГБ), GPU GTX 1650; Gazebo GUI через WSLg |

2. Розгортання та запуск

Створено окремий дистрибутив Ubuntu-22.04 (існуючий WSL користувача не змінювався). Скрипт setup_inside.sh встановлює ROS 2 Humble, Gazebo Harmonic, збирає у робочому просторі /opt/ardu_ws пакети ArduPilot (ardupilot_gz, micro-ROS agent, ros_gz, sdformat_urdf) і ArduPilot SITL. Під час збирання ros_gz_bridge вичерпувалась пам’ять (OOM), тому використано MAKEFLAGS=-j1 і --parallel-workers 1. Для коректного завантаження моделі дрона в libsdformat12 шляхи package:// у launch-файлі iris_lidar.launch.py замінено на model:// (SDF_PATH).

Запуск (три команди у каталозі проєкту):

./scripts/start_sim.sh        # Gazebo + SITL + micro-ROS agent

./scripts/build_and_run.sh    # colcon build + ros2 run fpv_lab2 flight_test

./scripts/stop_sim.sh         # зупинка всього

Приклад викладача запускається командою ros2 run fpv_lab2 flight_test (перевірка готовності → GUIDED → arm → зліт на 2 м → зависання → посадка); у моїй версії між зависанням і посадкою додано стан NAVIGATING.

3. Скрипти швидкого запуску

scripts/start_sim.sh — запускає симуляцію у фоні (лог ~/sim_logs/sim.log), задає точку спавну дрона й чекає на сервіси ArduPilot (є процес arducopter і сервіс /ap/v1/arm_motors).

#!/usr/bin/env bash

# Starts ArduPilot SITL + Gazebo (maze world) + ROS 2 bridge + DDS agent in the background.

# Usage:  ./scripts/start_sim.sh          (waits until /ap/v1 services appear)

source /opt/ros/humble/setup.bash

source /opt/ardu_ws/install/setup.bash

export GZ_VERSION=harmonic

export PATH="$HOME/.local/bin:$PATH"   # mavproxy.py lives here

# libsdformat12 (used by sdformat_urdf on Humble) resolves model:// through SDF_PATH

export SDF_PATH="$SDF_PATH:/opt/ardu_ws/install/ardupilot_gz_description/share/ardupilot_gz_description/models"

LOGDIR="$HOME/sim_logs"; mkdir -p "$LOGDIR"

if pgrep -f "iris_maze.launch.py" >/dev/null; then

echo "Simulation already running. Use ./scripts/stop_sim.sh first."; exit 1

fi

# RViz is not needed for the lab and uses a lot of RAM: disable it if the launch file allows

# Spawn point inside a free corridor of the maze (the straight path to the

# target point must not cross the 4 m high walls). Override: SPAWN_X / SPAWN_Y.

ARGS="x:=${SPAWN_X:-6.0} y:=${SPAWN_Y:--2.2}"

if ros2 launch ardupilot_gz_bringup iris_maze.launch.py --show-args 2>/dev/null | grep -q "rviz"; then

ARGS="$ARGS rviz:=false"

fi

echo "[start_sim] launching iris_maze $ARGS ..."

nohup ros2 launch ardupilot_gz_bringup iris_maze.launch.py $ARGS > "$LOGDIR/sim.log" 2>&1 &

echo $! > "$LOGDIR/sim.pid"

echo "[start_sim] waiting for ArduPilot ROS 2 services (up to 120 s) ..."

for i in $(seq 1 60); do

if pgrep -x arducopter >/dev/null && ros2 service list 2>/dev/null | grep -q "/ap/v1/arm_motors"; then

echo "[start_sim] ready. Log: $LOGDIR/sim.log"; exit 0

fi

sleep 2

done

echo "[start_sim] services did not appear, see $LOGDIR/sim.log"; exit 2

scripts/stop_sim.sh — зупиняє програму польоту, ros2 launch, Gazebo, SITL, DDS-агент та допоміжні вузли.

#!/usr/bin/env bash

# Stops everything started by start_sim.sh (ROS 2 launch, Gazebo, SITL, DDS agent).

echo "[stop_sim] stopping ..."

pkill -f "fpv_lab2/flight_test" 2>/dev/null; pkill -f "flight_test --ros-args" 2>/dev/null

pkill -INT -f "ros2 launch ardupilot_gz_bringup" 2>/dev/null

sleep 3

for p in "gz sim" ruby arducopter micro_ros_agent mavproxy rviz2 robot_state_publisher parameter_bridge; do

pkill -f "$p" 2>/dev/null

done

sleep 1

rm -f "$HOME/sim_logs/sim.pid"

echo "[stop_sim] left processes (should be empty):"

pgrep -fa "gz sim|arducopter|micro_ros_agent|ardupilot_gz" || echo "  none"

scripts/build_and_run.sh — синхронізує проєкт на швидку файлову систему Linux, збирає пакет fpv_lab2 і запускає flight_test із записом логу.

#!/usr/bin/env bash

# Syncs the project from Windows (D:) to the fast Linux filesystem, builds fpv_lab2 and runs the flight.

# Usage: ./scripts/build_and_run.sh [-p delta_x:=5.2 ...]   (ROS parameters are optional)

set -e

SRC="$(cd "$(dirname "$0")/.." && pwd)"

DST="$HOME/fpv_labs"

mkdir -p "$DST"

rsync -a -c --delete --exclude build --exclude install --exclude log --exclude .git --exclude __pycache__ "$SRC/" "$DST/"

source /opt/ros/humble/setup.bash

source /opt/ardu_ws/install/setup.bash

cd "$DST"

colcon build --symlink-install --packages-select fpv_lab2

source "$DST/install/setup.bash"

mkdir -p "$HOME/sim_logs"

ros2 run fpv_lab2 flight_test --ros-args "$@" 2>&1 | tee "$HOME/sim_logs/flight_$(date +%H%M%S).log"

4. Структура проєкту

fpv_labs/

├── README.md

├── scripts/            start_sim.sh, stop_sim.sh, build_and_run.sh

├── logs/               flight_variant23.log (лог звітного польоту)

└── src/fpv_lab2/       ROS 2 пакет (ament_python)

├── package.xml, setup.py, setup.cfg

├── test/test_navigation.py            (модульні тести навігаційної математики)

└── fpv_lab2/flight_test/

├── main.py                        точка входу

├── flight_test_node.py            вузол: підписки, клієнти сервісів, конфіг, publish_velocity

├── flight_state.py                enum станів (+ NAVIGATING)

├── navigation.py                  [нове] чиста математика: ціль, відстань, профіль швидкості, похибка

└── handlers/                      по одному класу-обробнику на стан

├── waiting_for_system.py, prearm_check.py, setting_guided.py, arming.py

├── taking_off.py, hovering.py

├── navigating.py              [нове] власний алгоритм польоту до цілі

└── landing.py, waiting_for_landing.py

5. Алгоритм польоту

Початкова точка (x0, y0) запам’ятовується за першим повідомленням /odometry (істинна позиція Gazebo); ціль: x_t = x0 − 5.2, y_t = y0 − 0.8.

Додано новий стан NAVIGATING і клас NavigatingHandler (базові класи ROS/ArduPilot не змінювались); перехід HOVERING → NAVIGATING → LANDING.

У кожному такті (0.2 с) обчислюється відстань d до цілі та одиничний вектор напрямку; швидкість v = clamp(K·d, v_min, v_max) з K = 0.4 с⁻¹, v_max = 0.4 м/с, v_min = 0.1 м/с — тобто лінійне гальмування на підльоті. Команда (vx, vy, 0) публікується як geometry_msgs/TwistStamped у /ap/v1/cmd_vel з frame_id = "map" (ENU; ArduPilot приймає лише "map" та "base_link").

Висоту утримує ArduPilot у режимі GUIDED (vz = 0). Коли d ≤ 0.2 м, горизонтальна швидкість обнуляється, фіксуються x, y і похибка e, 10 с дрон утримує позицію, після чого вмикається режим LAND (mode 9).

Аварійний вихід: якщо ціль не досягнута за nav_timeout, логується попередження і виконується така ж посадка. Після підтвердження посадки (висота Gazebo ≈ 0) програма логує фінальну позицію й коректно завершується.

Спостереження: кожна зміна позиції журналюється раз на секунду (pos, dist, speed).

6. Координати та похибка

| Величина | Значення |
| --- | --- |
| Початкова точка (x0, y0) | (6.000; −2.200) м |
| Цільова точка (x_t, y_t) | (0.800; −3.000) м |
| Фактична позиція перед посадкою (x, y) | (0.942; −2.976) м, висота 2.190 м |
| Похибка e = √((x−x_t)²+(y−y_t)²) | 0.144 м (допуск 0.20 м — виконано) |
| Фінальна позиція після посадки | (0.936; −2.976) м, висота 0.19 м (на землі); відхилення від цілі ≈ 0.138 м |
| Посадка | успішна («Landing completed», «Flight test completed successfully») |

Примітка щодо точки старту. У світі iris_maze за замовчуванням дрон з’являється в (0, 0), де за ≈1 м на захід розташована стіна лабіринту висотою 4 м (Wall_9, x = −1). Політ до цілі (−5.2; −0.8) від цієї точки неминуче врізається в стіну (так закінчились перші спроби — спрацював Crash-check ArduPilot). Тому дрон створюється у вільному коридорі лабіринту в точці (6.0; −2.2); цільова точка (0.8; −3.0) також лежить у цьому коридорі, шлях вільний. Формули варіанту залишено без змін — ціль рахується від фактичної старту.

7. Додаток коду: новий і змінений код

navigation.py (нове):

"""Pure navigation math (no ROS dependencies) so it can be unit-tested."""

import math

def compute_target(x0, y0, delta_x, delta_y):

"""Variant formula: target = start - delta."""

return x0 - delta_x, y0 - delta_y

def distance_and_direction(x, y, x_target, y_target):

"""Return (distance, unit_x, unit_y) from the current point to the target."""

dx = x_target - x

dy = y_target - y

dist = math.hypot(dx, dy)

if dist < 1e-9:

return 0.0, 0.0, 0.0

return dist, dx / dist, dy / dist

def speed_profile(dist, max_speed, gain, min_speed):

"""Proportional speed with saturation: slows down while approaching."""

return max(min(max_speed, gain * dist), min_speed)

def position_error(x, y, x_target, y_target):

"""e = sqrt((x - x_t)^2 + (y - y_t)^2)  (formula from section 2.8)."""

return math.hypot(x - x_target, y - y_target)

handlers/navigating.py (нове):

import time

from typing import TYPE_CHECKING

from ..flight_state import FlightState

from ..navigation import distance_and_direction, position_error, speed_profile

from .handler import FlightStateHandler

if TYPE_CHECKING:

from ..flight_test_node import FlightTestNode

# The drone must stay inside the tolerance for this long (s) before the

# target is considered reached (filters out a fly-through and the inertia).

SETTLE_TIME = 2.0

LOG_PERIOD = 1.0

class NavigatingHandler(FlightStateHandler):

"""Flight to (x_target, y_target) with a proportional velocity law.

v = clamp(K * distance, v_min, v_max), direction = unit vector to target.

Altitude is held with a small proportional vertical velocity.

"""

def __init__(self):

self._last_log = 0.0

self._hold_until = None

def handle(self, node: "FlightTestNode") -> None:

if node.status is None or not node.status.armed:

node.fail("Drone disarmed unexpectedly during navigation")

return

# After the result is logged, hold position for a while (screenshot time)

if self._hold_until is not None:

node.publish_velocity(0.0, 0.0, 0.0)

if time.monotonic() >= self._hold_until:

self._hold_until = None

node.get_logger().info("Hold at target finished, landing")

node.state = FlightState.LANDING

return

if node.odometry is None or node.target is None:

return

now = time.monotonic()

if node.nav_started_at is None:

node.nav_started_at = now

node.get_logger().info(

"Navigation started, target "

f"({node.target[0]:.3f}, {node.target[1]:.3f})"

)

p = node.odometry.pose.pose.position

x_t, y_t = node.target

tolerance = node.get_parameter("tolerance").value

v_max = node.get_parameter("max_speed").value

gain = node.get_parameter("speed_gain").value

v_min = node.get_parameter("min_speed").value

timeout = node.get_parameter("nav_timeout").value

dist, ux, uy = distance_and_direction(p.x, p.y, x_t, y_t)

# Vertical velocity is kept at zero: ArduPilot holds the altitude in

# GUIDED velocity control (a proportional z-term diverged because of

# the sign convention of the DDS velocity command).

vz = 0.0

if dist <= tolerance:

# Inside the tolerance: stop the horizontal motion.

node.publish_velocity(0.0, 0.0, vz)

if node.nav_arrived_since is None:

node.nav_arrived_since = now

if now - node.nav_arrived_since >= SETTLE_TIME:

self._finish(node, p, reached=True)

self._hold_until = now + node.get_parameter("target_hold").value

return

node.nav_arrived_since = None

if now - node.nav_started_at > timeout:

node.publish_velocity(0.0, 0.0, 0.0)

node.get_logger().warning(

f"Navigation timeout ({timeout:.0f} s), landing here")

self._finish(node, p, reached=False)

self._hold_until = now + node.get_parameter("target_hold").value

return

speed = speed_profile(dist, v_max, gain, v_min)

node.publish_velocity(speed * ux, speed * uy, vz)

if now - self._last_log >= LOG_PERIOD:

self._last_log = now

node.get_logger().info(

f"pos=({p.x:.2f}, {p.y:.2f}, {p.z:.2f}) "

f"dist={dist:.2f} m speed={speed:.2f} m/s"

)

@staticmethod

def _finish(node: "FlightTestNode", p, reached: bool) -> None:

x0, y0 = node.start_position

x_t, y_t = node.target

err = position_error(p.x, p.y, x_t, y_t)

node.publish_velocity(0.0, 0.0, 0.0)

node.get_logger().info(

"Target point reached" if reached else "Target point NOT reached")

node.get_logger().info(f"RESULT start    : x0={x0:.3f}, y0={y0:.3f}")

node.get_logger().info(f"RESULT target   : x={x_t:.3f}, y={y_t:.3f}")

node.get_logger().info(

f"RESULT actual   : x={p.x:.3f}, y={p.y:.3f}, z={p.z:.3f}")

node.get_logger().info(f"RESULT error    : e={err:.3f} m")

node.get_logger().info("Holding position at the target before landing")

flight_state.py (змінено: додано стан):

from enum import Enum, auto

class FlightState(Enum):

WAITING_FOR_SYSTEM = auto()

PREARM_CHECK = auto()

SETTING_GUIDED = auto()

ARMING = auto()

TAKING_OFF = auto()

HOVERING = auto()

NAVIGATING = auto()   # added in lab 2: flight to the target point

LANDING = auto()

WAITING_FOR_LANDING = auto()

FINISHED = auto()

ERROR = auto()

hovering.py (змінено: очікування висоти, перехід у NAVIGATING):

import time

from typing import TYPE_CHECKING

from ..flight_state import FlightState

from .handler import FlightStateHandler

if TYPE_CHECKING:

from ..flight_test_node import FlightTestNode

class HoveringHandler(FlightStateHandler):

"""Waits for the takeoff altitude, then hovers for HOVER_DURATION."""

STATUS_GRACE = 8.0       # s, status topic may lag right after takeoff

CLIMB_TIMEOUT = 300.0    # s, the simulation runs slower than real time

ALTITUDE_FRACTION = 0.9  # fraction of the takeoff altitude

def __init__(self) -> None:

self._reached_at = None

self._last_log = 0.0

def handle(self, node: "FlightTestNode") -> None:

now = time.monotonic()

since_takeoff = now - node.hover_started_at

if since_takeoff > self.STATUS_GRACE and (

node.status is None or not node.status.armed

):

node.fail("Drone disarmed unexpectedly during flight")

return

altitude = (

node.odometry.pose.pose.position.z

if node.odometry is not None else 0.0

)

if self._reached_at is None:

if altitude >= self.ALTITUDE_FRACTION * node.TAKEOFF_ALTITUDE:

self._reached_at = now

node.get_logger().info(

f"Takeoff altitude reached: {altitude:.2f} m"

)

elif since_takeoff > self.CLIMB_TIMEOUT:

node.fail("Takeoff altitude was not reached in time")

elif now - self._last_log > 5.0:

self._last_log = now

node.get_logger().info(f"Climbing, altitude {altitude:.2f} m")

return

if now - self._reached_at >= node.HOVER_DURATION:

node.get_logger().info("Hover completed, flying to the target point")

node.state = FlightState.NAVIGATING

prearm_check.py (змінено: повторні спроби та очікування EKF):

import time

from functools import partial

from typing import TYPE_CHECKING

from std_srvs.srv import Trigger

from ..flight_state import FlightState

from .handler import FlightStateHandler

if TYPE_CHECKING:

from ..flight_test_node import FlightTestNode

class PrearmCheckHandler(FlightStateHandler):

# The EKF needs some time after a fresh SITL start (slow simulation),

# so a negative answer is retried for a while before the test fails.

RETRY_TIMEOUT = 300.0

RETRY_PERIOD = 3.0

EKF_SETTLE = 45.0     # s (wall) after the first position estimate

EKF_TIMEOUT = 60.0    # s (wall) after the pre-arm check passed

def __init__(self) -> None:

self._passed = False

self._passed_at = 0.0

self._last_log = 0.0

self._first_try = None

self._next_try = 0.0

def handle(self, node: "FlightTestNode") -> None:

now = time.monotonic()

if self._first_try is None:

self._first_try = now

if self._passed:

self._wait_for_ekf(node, now)

elif not node.request_in_progress and now >= self._next_try:

self.call_prearm_check(node)

def _wait_for_ekf(self, node: "FlightTestNode", now: float) -> None:

# Arming too early makes the EKF re-align its yaw in flight (the

# slow simulation delays GPS), which crashed the drone. Wait for the

# position estimate plus a settling time.

got = node.geopose_first_at

if got is not None and now - got >= self.EKF_SETTLE:

node.state = FlightState.SETTING_GUIDED

elif now - self._passed_at >= self.EKF_TIMEOUT:

node.get_logger().warn("EKF wait timed out, continuing")

node.state = FlightState.SETTING_GUIDED

elif now - self._last_log > 10.0:

self._last_log = now

node.get_logger().info("Waiting for the EKF to settle before arming...")

def call_prearm_check(self, node: "FlightTestNode") -> None:

node.request_in_progress = True

request = Trigger.Request()

future = node.prearm_client.call_async(request)

future.add_done_callback(partial(self.prearm_callback, node))

node.get_logger().info("Running pre-arm check")

def prearm_callback(self, node: "FlightTestNode", future) -> None:

node.request_in_progress = False

try:

response = future.result()

except Exception as error:

node.fail(f"Pre-arm service failed: {error}")

return

if not response.success:

if time.monotonic() - self._first_try < self.RETRY_TIMEOUT:

node.get_logger().warn(

f"Not armable yet ({response.message}), retrying...")

self._next_try = time.monotonic() + self.RETRY_PERIOD

return

node.fail(f"Vehicle is not armable: {response.message}")

return

node.get_logger().info(f"Pre-arm check passed: {response.message}")

self._passed = True

self._passed_at = time.monotonic()

flight_test_node.py (змінено: параметри, одометрія, publish_velocity):

import time

from rclpy.node import Node

from geometry_msgs.msg import TwistStamped

from geographic_msgs.msg import GeoPoseStamped

from nav_msgs.msg import Odometry

from std_srvs.srv import Trigger

from ardupilot_msgs.msg import Status

from ardupilot_msgs.srv import ArmMotors, ModeSwitch, Takeoff

from .flight_state import FlightState

from .handlers import HANDLERS

from .navigation import compute_target

class FlightTestNode(Node):

GUIDED_MODE = 4

LAND_MODE = 9

TAKEOFF_ALTITUDE = 2.0

HOVER_DURATION = 20.0      # s (wall time) hover before the horizontal flight; sim runs slowly in WSL

def __init__(self):

super().__init__("flight_test")

# Individual task: variant 23 (delta_x = 5.2 m, delta_y = 0.8 m)

self.declare_parameter("delta_x", 5.2)

self.declare_parameter("delta_y", 0.8)

self.declare_parameter("max_speed", 0.4)       # m/s

self.declare_parameter("speed_gain", 0.4)      # 1/s

self.declare_parameter("min_speed", 0.10)      # m/s

self.declare_parameter("tolerance", 0.20)      # m

self.declare_parameter("nav_timeout", 600.0)   # s (wall time)

self.declare_parameter("target_hold", 10.0)    # s, hover at target before landing

self.declare_parameter("cmd_vel_topic", "/ap/v1/cmd_vel")

self.declare_parameter("cmd_vel_frame", "map")   # ArduPilot DDS accepts only "map" (ENU) or "base_link"

self.delta_x = self.get_parameter("delta_x").value

self.delta_y = self.get_parameter("delta_y").value

self.state = FlightState.WAITING_FOR_SYSTEM

self.request_in_progress = False

self.status = None

self.odometry = None

self.hover_started_at = None

self.geopose_first_at = None   # first filtered GeoPose = EKF has a position

self.start_position = None     # (x0, y0) from the first odometry

self.target = None             # (x_target, y_target)

self.nav_started_at = None

self.nav_arrived_since = None

self._handlers = HANDLERS

self.status_subscription = self.create_subscription(

Status, "/ap/v1/status", self.status_callback, 10)

self.odometry_subscription = self.create_subscription(

Odometry, "/odometry", self.odometry_callback, 10)

self.geopose_subscription = self.create_subscription(

GeoPoseStamped, "/ap/v1/geopose/filtered", self.geopose_callback, 10)

self.cmd_vel_publisher = self.create_publisher(

TwistStamped, self.get_parameter("cmd_vel_topic").value, 10)

self.prearm_client = self.create_client(Trigger, "/ap/v1/prearm_check")

self.mode_client = self.create_client(ModeSwitch, "/ap/v1/mode_switch")

self.arm_client = self.create_client(ArmMotors, "/ap/v1/arm_motors")

self.takeoff_client = self.create_client(

Takeoff, "/ap/v1/experimental/takeoff")

self.timer = self.create_timer(0.2, self.control_loop)

self.get_logger().info("Flight test node started")

self.get_logger().info("Waiting for ArduPilot and ROS services...")

def status_callback(self, message):

self.status = message

def geopose_callback(self, message):

if self.geopose_first_at is None:

self.geopose_first_at = time.monotonic()

self.get_logger().info("EKF position estimate is available")

def odometry_callback(self, message):

self.odometry = message

if self.start_position is None:

p = message.pose.pose.position

self.start_position = (p.x, p.y)

self.target = compute_target(p.x, p.y, self.delta_x, self.delta_y)

self.get_logger().info(

f"Start position: x0={p.x:.3f}, y0={p.y:.3f}")

self.get_logger().info(

f"Target point (variant 23): x={self.target[0]:.3f}, "

f"y={self.target[1]:.3f}  "

f"(delta_x={self.delta_x}, delta_y={self.delta_y})")

def services_ready(self):

return (

self.prearm_client.service_is_ready()

and self.mode_client.service_is_ready()

and self.arm_client.service_is_ready()

and self.takeoff_client.service_is_ready()

)

def control_loop(self):

handler = self._handlers.get(self.state)

if handler is not None:

handler.handle(self)

def publish_velocity(self, vx, vy, vz=0.0):

msg = TwistStamped()

msg.header.stamp = self.get_clock().now().to_msg()

msg.header.frame_id = self.get_parameter("cmd_vel_frame").value

msg.twist.linear.x = float(vx)

msg.twist.linear.y = float(vy)

msg.twist.linear.z = float(vz)

self.cmd_vel_publisher.publish(msg)

def call_mode_switch(self, mode, callback):

self.request_in_progress = True

request = ModeSwitch.Request()

request.mode = mode

future = self.mode_client.call_async(request)

future.add_done_callback(callback)

self.get_logger().info(f"Requesting flight mode {mode}")

def fail(self, message):

self.get_logger().error(message)

self.state = FlightState.ERROR

Також: waiting_for_system.py (додатково чекає першу одометрію) та waiting_for_landing.py (лог фінальної позиції) — див. репозиторій.

8. Лог звітного польоту (скорочено: повтори перед-старт перевірки прибрано, позиційні рядки прорідено)

[INFO] [1792705256] Flight test node started

[INFO] [1792705256] Waiting for ArduPilot and ROS services...

[WARN] [1792705256] New publisher discovered on topic '/ap/v1/geopose/filtered', offering incompatible QoS. No messages will be received from it. Last incompatible policy: RELIABILITY

[INFO] [1792705256] Start position: x0=6.000, y0=-2.200

[INFO] [1792705256] Target point (variant 23): x=0.800, y=-3.000  (delta_x=5.2, delta_y=0.8)

[INFO] [1792705257] ArduPilot and all services are available

[INFO] [1792705257] Running pre-arm check

[INFO] [1792705257] Pre-arm check passed: Vehicle is Armable

[INFO] [1792705257] Waiting for the EKF to settle before arming...

[WARN] [1792705317] EKF wait timed out, continuing

[INFO] [1792705317] Requesting flight mode 4

[INFO] [1792705317] GUIDED mode enabled

[INFO] [1792705317] Requesting motor arming

[INFO] [1792705317] Motors armed

[INFO] [1792705318] Requesting takeoff to 2.0 m

[INFO] [1792705318] Takeoff command accepted

[INFO] [1792705318] Hovering for 20.0 seconds

[INFO] [1792705318] Climbing, altitude 0.19 m

[INFO] [1792705348] Takeoff altitude reached: 1.82 m

[INFO] [1792705368] Hover completed, flying to the target point

[INFO] [1792705368] Navigation started, target (0.800, -3.000)

[INFO] [1792705368] pos=(6.00, -2.20, 2.19) dist=5.26 m speed=0.40 m/s

[INFO] [1792705375] pos=(5.55, -2.26, 2.20) dist=4.81 m speed=0.40 m/s

[INFO] [1792705382] pos=(4.86, -2.37, 2.19) dist=4.11 m speed=0.40 m/s

[INFO] [1792705389] pos=(4.14, -2.48, 2.19) dist=3.38 m speed=0.40 m/s

[INFO] [1792705396] pos=(3.45, -2.59, 2.19) dist=2.69 m speed=0.40 m/s

[INFO] [1792705403] pos=(2.73, -2.70, 2.19) dist=1.95 m speed=0.40 m/s

[INFO] [1792705410] pos=(1.97, -2.82, 2.19) dist=1.18 m speed=0.40 m/s

[INFO] [1792705416] pos=(1.37, -2.91, 2.19) dist=0.58 m speed=0.23 m/s

[INFO] [1792705422] pos=(1.03, -2.96, 2.19) dist=0.24 m speed=0.10 m/s

[INFO] [1792705426] Target point reached

[INFO] [1792705426] RESULT start    : x0=6.000, y0=-2.200

[INFO] [1792705426] RESULT target   : x=0.800, y=-3.000

[INFO] [1792705426] RESULT actual   : x=0.942, y=-2.976, z=2.190

[INFO] [1792705426] RESULT error    : e=0.144 m

[INFO] [1792705426] Holding position at the target before landing

[INFO] [1792705436] Hold at target finished, landing

[INFO] [1792705436] Requesting flight mode 9

[INFO] [1792705436] LAND mode enabled

[INFO] [1792705462] Landing completed

[INFO] [1792705462] Final Gazebo position: x=0.936, y=-2.976, altitude=0.19 m

[INFO] [1792705462] Flight test completed successfully

Повний лог: fpv_labs/logs/flight_variant23_full.log у репозиторії.

9. Скріншоти

Рис.1 Gazebo Sim: дрон Iris (модель із мешами) на початку руху до цілі, камера слідкування; старт (6.0; −2.2)

Рис.2 Політ до цілі: дрон на висоті ≈2.2 м рухається коридором лабіринту зі швидкістю 0.4 м/с

Рис.3 Наближення до цілі (до цілі менше 1 м): швидкість зменшується за законом v = clamp(K·d, v_min, v_max)

Рис.4 Загальний вигляд лабіринту зверху: дрон біля цілі (0.94; −2.98), утримання позиції перед посадкою

Рис.5 Кінець: дрон приземлився біля цілі (0.94; −2.98); фактичні координати та похибка — у таблиці та в журналі

Додаткова інформація

Швидкість симуляції. У WSL2 Gazebo (лідар, рендер) працює на ≈10–20 % реального часу, тому всі таймаути в програмі задані за настінним часом, а зліт чекає досягнення висоти (до 2 м) перед відліком зависання.

Помилки й виправлення під час відлагодження: (1) ArduPilot ігнорує команди швидкості з frame_id, відмінним від "map"/"base_link"; (2) топік керування — /ap/v1/cmd_vel (а не /ap/cmd_vel); (3) передпольотна перевірка одразу після старту повертає «Not armable» — додано повтор; (4) регулятор висоти з vz ≠ 0 був небезпечним через умову знаків, тому vz = 0 (висоту тримає ArduPilot); (5) політ упирався у стіну в точці спавну за замовчуванням — змінено точку спавну.

Відображення дрона. Спершу меші моделі Iris не завантажувались (URI package://ardupilot_gazebo/... не резолвиться у WSL, дрон малювався каркасом); у моделі iris_with_standoffs URI замінено на абсолютні file:// — тепер дрон відображається повноцінною моделлю (скрипт setup/fix_mesh2.sh). На фізику та політ це не впливає.

Тести: 4 модульні тести navigation.py (python -m pytest) проходять.

Висновок

У ході роботи розгорнуто середовище Gazebo Harmonic + ArduPilot SITL + ROS 2 Humble у WSL2, створено скрипти запуску/зупинки, запущено приклад викладача та реалізовано власний стан NAVIGATING (пропорційний профіль швидкості, зупинка на цілі, утримання, автопосадка). Дрон долетів до цілі варіанту 23 (Δx = 5.2 м, Δy = 0.8 м) із похибкою 0.144 м (< 0.2 м), утримав позицію, автоматично приземлився й програма завершилась коректно. Отримано практичний досвід інтеграції ROS 2 з ArduPilot через DDS, керування швидкістю у системі координат map та аналізу причин аварійних завершень польоту (Crash-check, зіткнення зі стіною).